# Routing

## Question

1. Draw a simple network diagram showing three devices connected through a router and label the IP addresses.

## Answer

Example Diagram:

             Internet
                 |
        Router (WAN: 203.0.113.2)
        Router (LAN: 192.168.1.1)
        /        |         \
Phone        Laptop      Smart TV
192.168.1.10 192.168.1.20 192.168.1.30

The router connects the local devices to the Internet. Each device has a unique IP address on the LAN.

## Question

2. Simulate manual IP routing from your laptop to a friend's computer in another subnet.

## Answer

Example:
Laptop: 192.168.1.20
Default Gateway: 192.168.1.1
Friend's PC: 192.168.2.50

Step 1: The laptop checks its routing table.
Route: 192.168.2.0/24 -> Gateway 192.168.1.1

Step 2: The packet is sent to Router A (192.168.1.1).

Step 3: Router A checks its routing table.
Route: 192.168.2.0/24 -> Next Hop 10.0.0.2

Step 4: Router B receives the packet and forwards it to 192.168.2.50 because the destination network is directly connected.

## Question

3. Create a routing table for a fictional Zomato-style delivery network with three branches.

## Answer

| Network | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- |
| 10.1.0.0 | 255.255.255.0 | Direct | G0/0 |
| 10.2.0.0 | 255.255.255.0 | 10.1.0.2 | G0/1 |
| 10.3.0.0 | 255.255.255.0 | 10.1.0.3 | G0/2 |

## Question

4. Compare static routing and dynamic routing protocols (RIP/OSPF).

## Answer

Static Routing
Pros:
• Simple for small networks.
• Low CPU and bandwidth usage.
• More secure because routes are manually controlled.

Cons:
• Difficult to manage in large networks.
• No automatic updates.
• Manual changes are required after network failures.

Dynamic Routing (RIP/OSPF)
Pros:
• Automatically learns routes.
• Better for large networks.
• Quickly adapts to network changes.

Cons:
• More complex to configure.
• Uses CPU and bandwidth.
• Requires routing protocol support.

Example: A food delivery app with many branches benefits from OSPF because routes update automatically if a link fails.

## Question

5. Use AI to generate an OSPF configuration and explain each line.

## Answer

Sample Configuration:

router ospf 1
 network 192.168.1.0 0.0.0.255 area 0
 network 10.0.0.0 0.0.0.3 area 0

Explanation:
• router ospf 1 - Starts OSPF process ID 1.
• network 192.168.1.0 0.0.0.255 area 0 - Enables OSPF for the LAN network.
• network 10.0.0.0 0.0.0.3 area 0 - Enables OSPF on the router-to-router link in Area 0.