# IPv6 Configuration

## Question

1. Open your Windows or Linux system, use the command line to assign a static IPv6 address to your primary network adapter, and verify the configuration using ipconfig (Windows) or ifconfig/ip (Linux).

## Answer

Windows:
1. Open Command Prompt as Administrator.
2. Run:
netsh interface ipv6 set address "Ethernet" static 2001:db8:1::10
3. Verify using:
ipconfig

Linux:
sudo ip -6 addr add 2001:db8:1::10/64 dev eth0
ip -6 addr show

Result: Verify that the configured IPv6 address appears on the network adapter.

## Question

2. Configure a unique local IPv6 address (starting with 'fd') on your system and confirm connectivity by pinging another device on your LAN.

## Answer

Example unique local address:
fd00:1234:5678:1::10/64

Assign the address using the operating system's network command.
Verify using:
Windows: ping -6 fd00:1234:5678:1::20
Linux: ping6 fd00:1234:5678:1::20

Successful replies confirm IPv6 connectivity.

## Question

3. Set up a temporary IPv6 address on your system and check if it changes after a reboot or reconnect.

## Answer

Steps:
1. Enable temporary IPv6 addresses.
2. Disconnect and reconnect the network or restart the computer.
3. Check the IPv6 address using ipconfig or ip -6 addr show.

Observation:
The temporary IPv6 address changed after reconnecting, while the permanent IPv6 address remained the same.

## Question

4. Write a small script that searches restaurant names and returns the matching IPv6 address.

## Answer

Python Example:

restaurants = {
 'Pizza Hub':'fd00:1::10',
 'Spice Garden':'fd00:1::20',
 'Burger Point':'fd00:1::30'
}

name = input('Enter restaurant name: ').lower()
for r, ip in restaurants.items():
    if name in r.lower():
        print(r, ip)

## Question

5. Use AI to generate an IPv6 subnetting scenario and assign a valid address from one subnet.

## Answer

Sample Scenario:
Subnet 1: 2001:db8:100:1::/64
Subnet 2: 2001:db8:100:2::/64
Subnet 3: 2001:db8:100:3::/64

Assigned Address:
2001:db8:100:2::25/64

Configuration Steps:
1. Open network settings or Command Prompt.
2. Assign the IPv6 address 2001:db8:100:2::25/64.

3. Verify using ipconfig (Windows) or ip -6 addr show (Linux).