# OSI 7-Layer Model

## Question 1

Draw a diagram of the OSI 7-layer model and label each layer with its name and a real-world example.

### Answer:

| Layer | Example |
| --- | --- |
| 7. Application | HTTP, HTTPS, Browser |
| 6. Presentation | SSL/TLS Encryption |
| 5. Session | NetBIOS, API Session |
| 4. Transport | TCP, UDP |
| 3. Network | Router, IP |
| 2. Data Link | Switch, MAC Address |
| 1. Physical | Cables, Hub, Electrical Signals |

## Question 2

### Answer:

- Encrypting data before sending: Presentation Layer (Layer 6)

- Routing packets across networks: Network Layer (Layer 3)

- Converting data to electrical signals: Physical Layer (Layer 1)

- Opening a web page in your browser: Application Layer (Layer 7)

- Detecting and correcting errors in frames: Data Link Layer (Layer 2)

## Question 3

Pick any app and describe how at least three OSI layers are involved.

### Answer (WhatsApp):

Application Layer (Layer 7) allows me to send and receive messages using WhatsApp. Transport Layer (Layer 4) uses TCP or UDP to deliver data reliably. Network Layer (Layer 3) routes packets through the Internet to WhatsApp servers. Finally, the Data Link and Physical layers transmit the data over Wi-Fi or mobile networks.

## Question 4

You can ping a website's IP address but cannot open the website. Which OSI layer may be causing the problem?

### Answer:

If the IP address responds to ping, the Physical, Data Link, and Network layers are working correctly. The problem is most likely at the Application Layer (Layer 7), where services such as HTTP or HTTPS are used. It could also involve the Presentation Layer (Layer 6) if there is an SSL/TLS certificate or encryption issue.