# Day 5: Network Testing Tools

## Question

1. Use ping command to test connectivity to www.youtube.com and note average response time.

## Answer

Command:
ping www.youtube.com

Example Result:
Packets sent = 4, received = 4
Average response time = 30–60 ms (varies by network)

This shows the network is reachable and measures latency (speed of response).

## Question

2. Run traceroute to www.flipkart.com and list hops.

## Answer

Command:
tracert www.flipkart.com

Example Hops:
1. Local Router (192.168.1.1)
2. ISP Gateway
3. Core Network Router
4. Regional Router
5. Flipkart CDN Server

Each hop represents a device that forwards the packet toward the destination.

## Question

3. Use nslookup to find IP address of www.zomato.com.

## Answer

Command:
nslookup www.zomato.com

Example Output:
Name: www.zomato.com
Address: 192.168.157.2
This shows the IP address returned by DNS for the domain name.

## Question

4. Test connectivity to another device using ping and explain offline case.

## Answer

Command:
ping 192.168.1.5

If device is online:
Reply from 192.168.1.5: bytes=32 time<1ms TTL=64

If device is offline:
Request timed out.

This means the device is not reachable or is turned off.

## Question

5. Short report on ping, traceroute, nslookup usage.

## Answer

(a) Website not loading → traceroute helps identify where the connection fails (e.g., ISP or server hop).

(b) Slow connection → ping helps measure delay. Example: high ping time (>100ms) means slow network.

(c) DNS issues → nslookup checks if domain name resolves to correct IP. Example: if www.zomato.com does not return IP, DNS is the issue.