# Day 3: Switch and Router Configuration Basics -

## Question

1. Create two VLANs (Sales and Support) with IDs 10 and 20 in Cisco Packet Tracer.

## Answer

Switch CLI Commands:

enable
configure terminal
vlan 10
name Sales
vlan 20
name Support

This creates two separate VLANs on the switch for logical segmentation.

## Question

2. Assign switch ports to VLANs and verify using show vlan command.

## Answer

Commands:

interface range fa0/1-3
switchport mode access
switchport access vlan 10

interface range fa0/4-5
switchport mode access
switchport access vlan 20

Verification:
show vlan brief

This displays VLAN membership and confirms correct port assignment.

## Question

3. Configure static routing between two routers.

## Answer

Example:

Router 1:
interface g0/0
ip address 192.168.1.1 255.255.255.0
no shutdown

ip route 192.168.2.0 255.255.255.0 10.0.0.2

Router 2:
interface g0/0
ip address 192.168.2.1 255.255.255.0
no shutdown

Static route allows Router 1 to reach Router 2's network.

## Question

4. VLAN communication issue explanation.

## Answer

Ping fails between Sales and Support VLAN because VLANs are separate broadcast domains and Layer 2 switch cannot route between them.

Solution:
Use router-on-a-stick (inter-VLAN routing) or Layer 3 switch.

Example:
Configure trunk port + subinterfaces on router to allow communication between VLAN 10 and VLAN 20.