# VLAN Configuration

## Question

1. Draw a simple network diagram showing three devices connected to a switch and label the VLANs.

## Answer

## Question

2. Configure a simulated switch with VLAN 10 and VLAN 20, assign PCs, and verify connectivity.

## Answer

Sample Cisco CLI Commands:

enable
configure terminal
vlan 10
name STREAMING
vlan 20
name PERSONAL

interface range fa0/1-3
switchport mode access
switchport access vlan 10

interface range fa0/4-5
switchport mode access
switchport access vlan 20

Test:
PCs in VLAN 10 can ping each other.
PCs in VLAN 20 can ping each other.
PCs in different VLANs cannot ping because no inter-VLAN routing is configured.

## Question

3. Move a PC from VLAN 10 to VLAN 20 and verify connectivity.

## Answer

Before:
interface fa0/3
switchport access vlan 10

After:
interface fa0/3
switchport access vlan 20

Result:
The PC can no longer communicate with VLAN 10 devices. It can now communicate with devices in VLAN 20 after receiving the correct IP configuration.

## Question

4. Write a command list to assign 10 ports to VLAN 30 using a range command.

## Answer

Sample Cisco CLI:

enable
configure terminal
vlan 30
name OFFICE

interface range fa0/1-10
switchport mode access
switchport access vlan 30
end
write memory

The 'interface range' command configures ports Fa0/1 to Fa0/10 together, avoiding repeated commands.