# Day 4: Firewall Basics -

## Question

1. Create a new outbound rule to block Spotify traffic using Windows Defender Firewall.

## Answer

Steps:
1. Open Windows Defender Firewall with Advanced Security.
2. Click on Outbound Rules.
3. Select 'New Rule'.
4. Choose 'Program' and click Next.
5. Browse and select Spotify.exe.
6. Select 'Block the connection'.
7. Apply the rule to Domain, Private, and Public profiles.
8. Name the rule 'Block Spotify Traffic' and finish.

This blocks Spotify from accessing the internet.

## Question

2. Allow inbound connections for Google Chrome on port 443 (HTTPS).

## Answer

Steps:
1. Open Windows Defender Firewall Advanced Settings.
2. Click Inbound Rules → New Rule.
3. Select 'Port'.
4. Choose TCP and enter port 443.
5. Select 'Allow the connection'.
6. Apply to all profiles.
7. Name the rule 'Allow Chrome HTTPS'.

This allows secure HTTPS traffic through firewall for Chrome.

## Question

3. Block traffic from a specific IP address (123.45.67.89).

## Answer

Steps:
1. Open Windows Defender Firewall with Advanced Security.
2. Go to Outbound Rules → New Rule.
3. Select 'Custom Rule'.
4. In Scope tab, under Remote IP address, enter 123.45.67.89.
5. Select 'Block the connection'.
6. Apply to all profiles.
7. Save rule.

This blocks all network communication with that IP address.

## Question

4. Compare hardware and software firewalls.

## Answer

Hardware Firewall:
- Installed between network and internet.
- Protects entire network.
- Used in offices and enterprises.

Software Firewall:
- Installed on individual devices.
- Protects single computer.
- Example: Windows Defender Firewall.

Differences:
1. Scope: Network vs Device level
2. Cost: Expensive vs Free/Low cost
3. Management: Centralized vs Individual control

Use case:
Hardware firewall protects college networks, while software firewall protects personal laptops.