# Day 3: Introduction to IP Addressing

## Question

1. List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

## Answer

| IPv4 | IPv6 |
| --- | --- |
| 32-bit address | 128-bit address |
| Uses dotted decimal notation | Uses hexadecimal notation |
| Limited address space | Very large address space |
| Supports NAT commonly | Usually does not require NAT |

## Question

2. Assign static IPv4 addresses to four devices using the 192.168.10.0/24 network.

## Answer

Device           IP Address       Subnet Mask        Default Gateway

Laptop            192.168.10.10    255.255.255.0      192.168.10.1
Phone             192.168.10.11    255.255.255.0      192.168.10.1
Smart TV          192.168.10.12    255.255.255.0      192.168.10.1
Printer           192.168.10.13    255.255.255.0      192.168.10.1

## Question

3. Calculate valid subnets and hosts per subnet using a /27 subnet mask on the 192.168.1.0 network.

## Answer

/27 = 255.255.255.224

Borrowed bits = 3
Valid subnets = 8
Usable hosts per subnet = 30

First three subnets:
1. Network: 192.168.1.0
   Broadcast: 192.168.1.31
   Hosts: 192.168.1.1 - 192.168.1.30

2. Network: 192.168.1.32
   Broadcast: 192.168.1.63
   Hosts: 192.168.1.33 - 192.168.1.62

3. Network: 192.168.1.64
   Broadcast: 192.168.1.95
   Hosts: 192.168.1.65 - 192.168.1.94

## Question

4. Determine the network address, broadcast address, and valid host range for IP address 10.0.5.17 with subnet mask 255.255.255.0.

## Answer

Subnet Mask = 255.255.255.0 = /24

Network Address: 10.0.5.0
Broadcast Address: 10.0.5.255
Valid Host Range: 10.0.5.1 to 10.0.5.254

## Question

5. Verify the previous answer using an online subnet calculator and explain whether the result matches.

## Answer

Example Answer:
I checked the values using an online subnet calculator. The network address, broadcast address, and host range matched my manual calculation.

Note: Attach your own screenshot from the subnet calculator with your assignment.